 TOYOTA RAV-4  2001(2005
[image: image1.png]Toyota Rav-4 2001-2005

1. Undip and remove side panel from
both sides of radio and climate controls.

(Figure A)

2. Remove (4) Phillips screws securing the
radio/dimate control trim panel. (Figure B)

3. Remove (4) 10mm bolts securing the
radio. (Figure O

4. Unplug and remove radio.

5. Remove factory brackets from radio. Retain
‘brackets for use during kit assembly.

Continue to Kit Preparation

B=_¢
=
GO

it

Fues)

Foued)





[image: image2.png]For the Toyota Rav-4 2001-2005, Toyota Highlander 2001-
2007 (without factory navigation), Toyota Tundra 2003-2006
and Toyota Sequoia 2003-2007:

Cut off the rear section of the IS0 DDIN trim plate brackets,
as shown by the dotted fines labeled “B” in. (Figure A)

Continue to Kit Assembly

Carefully cut-out the shaded area
using adremeltool to acommodate
If bracket is broken forthe factory bracket.
during trimming use
this portion. .- Style of factory

=77 bracketsmay differ.

(Figure A) (Fgure B)




[image: image3.png]150 DDIN radio provision
1. Ifapplicable, remove the trim ring from the aftermarket radio.

2. Attach the factory mounting brackets and the /SO DDIN trim
Pplate brackets to the radio with the screws supplied with the
radio.

Note: Remove locating pins from factory brackets using a
grinder tool

3. Locate the factory wiring harness in the dash. Metra
recommends using the proper mating adapter from Metra or
AXXESS. Test the radio for proper operation.

4. Reassemble the dash in reverse order of disassembly.

Style of factory brackets
may differ





